CPE301 – SPRING 2019

# Design Assignment 1

Student Name: Cody Jones

Student #: 5002863328

Student Email: Jonesc30@unlv.nevada.edu

Primary Github address: https://github.com/Jonesc30/Submission

Directory: Submission

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List of Components used

Block diagram with pins used in the Atmega328P

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

.org 0			; location for reset

jmp MAIN

.org 0x02		; location for external interrupt INT0

jmp EX0_ISR

MAIN:

ldi R20, HIGH(RAMEND)	; initialize stack

out SPH, R20

ldi R20, LOW(RAMEND)

out SPL, R20

ldi R20, 2		; make INT0 falling edge triggered

sts EICRA, R20

sbi DDRB, 3		; set PORTB.3 as output

sbi PORTB, 3	; set LED off

sbi PORTD, 3	; turn on pull-up

cbi DDRC, 3		; set PORTC.3 as input

sbi PORTC, 3	; enable pull up

ldi R20, 1<<INT0	; enable INT0

out EIMSK, R20

sei				; enable interrupts

WHILE:

cbi PORTB, 3	; set LED on

; Delay 4 000 000 cycles

; 250ms at 16.0 MHz

ldi  r18, 21

ldi  r19, 75

ldi  r20, 191

L1: dec  r20

brne L1

dec  r19

brne L1

dec  r18

brne L1

nop

sbi PORTB, 3	; set LED off

; Delay 6 000 000 cycles

; 375ms at 16.0 MHz

ldi  r18, 31

ldi  r19, 113

ldi  r20, 31

L2: dec  r20

brne L2

dec  r19

brne L2

dec  r18

brne L2

nop

jmp	WHILE		; wait for interrupt

EX0_ISR:

cbi PORTB, 3

; Delay 21 328 000 cycles

; 1s 333ms at 16.0 MHz

ldi  r18, 109

ldi  r19, 51

ldi  r20, 106

L1: dec  r20

brne L1

dec  r19

brne L1

dec  r18

brne L1

reti			; return from interrupt

#define F_CPU 16000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

int main(void)

{

DDRB = (1<<3);			// set PORTB.3 for output

PORTB = (1<<3);		// set LED low

PORTD = (1<<2);		//pull-up activated

EIMSK = (1<<INT0);		// enable external interrupt 0

EICRA = 0x2;			// make INT0 falling edge triggered

sei ();				// enable interrupts

while (1)

{

_delay_ms(250);	// delay for 40% DC

PORTB &= ~(1<<3);	// set LED on

_delay_ms(375);	// delay for remaining 60%

PORTB |= (1<<3);	// set LED off

}

}

ISR (INT0_vect)	{		// ISR for external interrupt INT0

PORTB &= ~(1<<3);		// turns on PORTB.3 (LED)

_delay_ms(1333);		// delay for 1.33 sec

}

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://www.youtube.com/watch?v=kvSSHEG52GU

- GITHUB LINK OF THIS DA

https://github.com/Jonesc30/Submission/tree/master/DesignAssignments

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

NAME OF THE STUDENT