CPE301 – FALL 2019

# Design Assignment 2C

Student Name: Cody Jones

Student #: 5002863328

Student Email: Jonesc30@unlv.nevada.edu

Primary Github address: https://github.com/Jonesc30/Submission

Directory: Submission

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List of Components used

Block diagram with pins used in the Atmega328P

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1

#define F_CPU 16000000UL

#include <avr/io.h>

int main(void)

{

DDRB = 0xFF;					//all registers in DDRB are outputs

DDRC = 0x00;					//all registers in DDRC are inputs

PORTB = 0xFF;					//turn all LEDs on

TCCR0A = 0x00;					//normal operation

TCNT0 = 0x00;					//start the timer

TCCR0B |= (1 << CS02) | (1 << CS00); //set prescaler to 1024

int count = 0;						//use this to count overflow

//The fout for this is about 60Hz

while (1)

{

count = 0;					// reset counter

TCNT0 = 0x00;				// reset timer

while (count < 23)			// loop until 60% DC is met

{

if(TCNT0 == 0xFF)		// if timer overflows

{

count++;			// increment counter

TCNT0 = 0x00;		// reset timer

}

else if(!(PINC & (1 << 3))) //check if SW3 is pushed

{

PORTB |= (1 << 3); //turn off LED @ PB3

PORTB &= ~(1 << 2); //turn on LED @ PB2

TCNT0 = 0x00; //reset timer if switch is pushed

count = 0;

while(count < 82) //1.333 seconds

{

if(TCNT0 == 0xFF)

{

count++;

TCNT0 = 0x00;

}

}

PORTB |= (1 << 2); //turn off LED @ PB2

}

}

PORTB ^= (1<<3);			// toggle LED off

count = 0;					// reset counter

TCNT0 = 0x00;				// reset timer

while (count < 15)			// loop until remaining 40% DC is met

{

if(TCNT0 == 0xFF)		// if timer overflows

{

count++;			// increment counter

TCNT0 = 0x00;		// reset timer

}

else if(!(PINC & (1 << 3))) //check if SW3 is pushed

{

PORTB |= (1 << 3); //turn off LED @ PB3

PORTB &= ~(1 << 2); //turn on LED @ PB2

TCNT0 = 0x00; //reset timer if switch is pushed

count = 0;

while(count < 82) //1.333 seconds

{

if(TCNT0 == 0xFF)

{

count++;

TCNT0 = 0x00;

}

}

PORTB |= (1 << 2); //turn off LED @ PB2

}

}

PORTB ^= (1<<3);			// toggle LED on

}

return 0;

}

INITIAL/MODIFIED/DEVELOPED CODE OF TASK 2

#define F_CPU 16000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

uint8_t OVFCOUNT = 0;		// overflow count multiplier

uint8_t OVFLIMIT = 16;		// overflow count limit, set for 40% of DC

int main(void)

{

DDRB = 0xFF;					//all registers in DDRB are outputs

DDRC = 0x00;					//all registers in DDRC are inputs

PORTB = 0xFF;					//turn all LEDs on

TCCR0A = 0x00;					//normal operation

TCNT0 = 12;					//start the timer for .625 s, 40 clock cycles

TCCR0B |= (1 << CS02) | (1 << CS00); //set prescaler to 1024

TIMSK0 = (1<<TOIE0);				//enable timer0 overflow interrupt

sei();								//enable interrupts

while (1)

{

}

}

ISR (TIMER0_OVF_vect)

{

OVFCOUNT++;			// increment counter

if (OVFCOUNT == OVFLIMIT)

{

PORTB ^= (1<<3);		// toggle LED

PORTB |=(1<<2);			// LED off from switch

if(OVFLIMIT == 16)

{

OVFLIMIT = 24;		// set ovf limit for 60% of DC

if(!(PINC & (1 << 3))) //check if SW3 is pushed

{

PORTB &= ~(1<<2);	//LED on

OVFLIMIT = 85;		//when switch is pressed it goes through 85 cycles for 1.3333 sec

PORTB |= (1<<3);	//LED off

}

}

else

{

OVFLIMIT = 16;		// set ovf limit for 40% of DC

if(!(PINC & (1 << 3))) //check if SW3 is pushed

{

PORTB &= ~(1<<2);	//LED on

OVFLIMIT = 85;		//when switch is pressed it goes through 85 cycles for 1.3333 sec

PORTB |= (1<<3);	//LED off

}

}

OVFCOUNT = 0;			// reset counter

}

TCNT0 = 12;			// timer value for .625 sec

}

INITIAL/MODIFIED/DEVELOPED CODE OF TASK 3

#define F_CPU 16000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

uint8_t OVFCOUNT = 0;		// overflow count multiplier

uint8_t OVFLIMIT = 16;		// overflow count limit, set for 40% of DC

int main(void)

{

DDRB = 0xFF;					//all registers in DDRB are outputs

DDRC = 0x00;					//all registers in DDRC are inputs

PORTB = 0xFF;					//turn all LEDs on

OCR0A = 244;					//TOP=256-12

TCNT0 = 0;						//start the timer

TCCR0A |= (1<<WGM01);

TCCR0B |= (1 << CS02) | (1 << CS00); //set prescaler to 1024, CTC mode

TIMSK0 |= (1<<OCIE0A);				//enable timer0 overflow interrupt

sei();								//enable interrupts

while (1)

{

}

}

ISR (TIMER0_COMPA_vect)

{

OVFCOUNT++;			// increment counter

if (OVFCOUNT == OVFLIMIT)

{

PORTB ^= (1<<3);		// toggle LED

PORTB |=(1<<2);			// LED off from switch

if(OVFLIMIT == 16)

{

OVFLIMIT = 24;		// set ovf limit for 60% of DC

if(!(PINC & (1 << 3))) //check if SW3 is pushed

{

PORTB &= ~(1<<2);	//LED on

OVFLIMIT = 85;		//when switch is pressed it goes through 85 cycles for 1.3333 sec

PORTB |= (1<<3);	//LED off

}

}

else

{

OVFLIMIT = 16;		// set ovf limit for 40% of DC

if(!(PINC & (1 << 3))) //check if SW3 is pushed

{

PORTB &= ~(1<<2);	//LED on

OVFLIMIT = 85;		//when switch is pressed it goes through 85 cycles for 1.3333 sec

PORTB |= (1<<3);	//LED off

}

}

OVFCOUNT = 0;			// reset counter

}

TCNT0 = 0;			// timer value

}

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

Task 1

Task 2

Task 3

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

Task 1: https://www.youtube.com/watch?v=Tqrw04tKO8w

Task 2: https://www.youtube.com/watch?v=E3nyYXMj7pA

Task 3: https://www.youtube.com/watch?v=z52RTvPqKpg

- GITHUB LINK OF THIS DA

https://github.com/Jonesc30/Submission/tree/master/DesignAssignments

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

NAME OF THE STUDENT