CPE301 – FALL 2019

# Design Assignment 3

Student Name: Cody Jones

Student #: 5002863328

Student Email: Jonesc30@unlv.nevada.edu

Primary Github address: https://github.com/Jonesc30/Submission

Directory: Submission

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmega328PB

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

#define F_CPU 16000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#include <stdio.h>

#include <stdlib.h>

#include <math.h>

#define BAUDRATE 9600

#define BAUD_PRESCALLER (((F_CPU / (BAUDRATE * 16UL))) - 1)

void USART_init(void); //function to initialize USART

unsigned char USART_receive(void); //function to receive through USART

void USART_transmit(unsigned char data); //function to send through USART

void USART_putstring(char* StringPtr); //function to scan through the string

float cj = 2.500; //float number chosen for assignment

char String[] = "DA3A"; //string for task

char newline[] = "\n"; //so that I can create a new line

char buffer[5]; //buffer created for the random int number that will be 5 digits

char cj_buffer[4]; //buffer created for float number chosen above

uint8_t OVF_COUNT = 0; //initialize the overflow count for interrupt

uint8_t OVF_LIMIT = 250; //set the limit the count can reach to set 1 sec delay

int main(void)

{

USART_init(); //initialize USART

TCCR0A = 0x00; //normal operation

TCCR0B |= (1 << CS02); //set prescalar to 256

TCNT0 = 16; //TOP = 256-250 = 16

TIMSK0 |= (1 << TOIE0);

sei(); //enable interrupt

while (1)

{ //sit here waiting for interrupt

}

}

void USART_init(void)

{

UBRR0H = (uint8_t)(BAUD_PRESCALLER >> 8);

UBRR0L = (uint8_t)(BAUD_PRESCALLER);

UCSR0B = (1 << RXEN0) | (1 << TXEN0);

UCSR0C = (3 << UCSZ00);

}

unsigned char USART_receive(void)

{

while(!(UCSR0A & (1 << RXC0)));

return UDR0;

}

void USART_transmit(unsigned char data)

{

while(!(UCSR0A & (1 << UDRE0)));

UDR0 = data;

}

void USART_putstring(char* StringPtr)

{

while(*StringPtr != 0x00)

{

USART_transmit(*StringPtr);

StringPtr++;

}

}

// C program for implementation of ftoa()

// reverses a string 'str' of length 'len'

void reverse(char *str, int len)

{

int i=0, j=len-1, temp;

while (i<j)

{

temp = str[i];

str[i] = str[j];

str[j] = temp;

i++; j--;

}

}

// Converts a given integer x to string str[].  d is the number

// of digits required in output. If d is more than the number

// of digits in x, then 0s are added at the beginning.

int intToStr(int x, char str[], int d)

{

int i = 0;

while (x)

{

str[i++] = (x%10) + '0';

x = x/10;

}

// If number of digits required is more, then

// add 0s at the beginning

while (i < d)

str[i++] = '0';

reverse(str, i);

str[i] = '\0';

return i;

}

// Converts a floating point number to string.

void ftoa(float n, char *res, int afterpoint)

{

// Extract integer part

int ipart = (int)n;

// Extract floating part

float fpart = n - (float)ipart;

// convert integer part to string

int i = intToStr(ipart, res, 0);

// check for display option after point

if (afterpoint != 0)

{

res[i] = '.';  // add dot

// Get the value of fraction part upto given no.

// of points after dot. The third parameter is needed

// to handle cases like 233.007

fpart = fpart * pow(10, afterpoint);

intToStr((int)fpart, res + i + 1, afterpoint);

}

}

ISR (TIMER0_OVF_vect)

{

OVF_COUNT++; //increment the overflow counter

if (OVF_COUNT == OVF_LIMIT) //check to see if the limit was reached

{

USART_putstring(String); //print string to serial monitor (terminal window)

USART_putstring(newline); //go to next line

itoa(rand(), buffer, 10); //convert random int number (base 10) to a string

USART_putstring(buffer); //print out the converted string stored in buffer

USART_putstring(newline); //go to next line

ftoa(cj, cj_buffer, 2); //convert float number (2 digits after decimal precision) to a string

USART_putstring(cj_buffer); //print out the converted string stored in buffer

USART_putstring(newline); //go to next line

USART_putstring(newline); //go to next line

OVF_COUNT = 0; //reset overflow counter

}

TCNT0 = 16; //reset TOP

}

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://www.youtube.com/watch?v=sY73ztCTLSw

- GITHUB LINK OF THIS DA

https://github.com/Jonesc30/Submission/tree/master/DesignAssignments

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Cody Jones