CPE301 – FALL 2019

# Design Assignment 4A

Student Name: Cody Jones

Student #: 5002863328

Student Email: Jonesc30@unlv.nevada.edu

Primary Github address: https://github.com/Jonesc30/Submission

Directory: Submission

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmega328PB and shield, FTDI, PWM DC Motor, wires and DC Motor Driver

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

#define F_CPU 16000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

volatile unsigned int ADC_Val;			// holds value of ADC

int toggle = 0;							// used to toggle

int main(void)

{

DDRD |= 0x40; //PWM output

DDRC = 0x02;

// PC0 as input and PC1 is now an output

PORTC |= (1<<PC1);

DDRB |= (1<<2);			//PB1 as output to stop the motor on and off

PORTB |= (1<<2);				//Stops and turns it on

TCCR0A = 0x83;

// set none-inverting mode

// set fast PWM Mode for timer0 max height is 256

TCCR0B = 0x05;

// set prescaler to 1024 and starts PWM

PCICR = (1<<PCIE1);			//Enable PCINT

PCMSK1 = (1<<PCINT9);		// Set interupt for PC1

//Set up ADC

ADMUX = (1<<REFS0);												// PC0 is input for ADC

ADCSRA |= (1<<ADEN) | (1<<ADSC) | (1<<ADATE) | (1<<ADPS2) | (1<<ADPS1) | (1<<ADPS0);		// enable ADC, system clock used for ADC

sei();		//enable interrupts

while (1);

}

//ISR for PC1

ISR(PCINT1_vect)

{

if(!(PINC & (1 << PINC1)))

{

if(toggle == 0)

{

OCR0A = 0;

PORTB &= ~(1 << 2);			//turn off OCR0A

}

else if(toggle == 1)

{

PORTB |= (1 << 2);				//turn on OCR0A

ADCSRA |= (1 << ADSC);				// start conversion

while((ADCSRA&(1<<ADIF))==0)

{	// wait for conversion to finish

ADC_Val = ADC & 0x03FF;			// we only need first 10 bits

OCR0A = 10*ADC_Val;				// Calculate duty cycle

}

}

toggle ^= 1;		//toggle

}

}

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://www.youtube.com/watch?v=YOYFHud2kk4

- GITHUB LINK OF THIS DA

https://github.com/Jonesc30/Submission/tree/master/DesignAssignments

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Cody Jones