CPE301 – FALL 2019

# Design Assignment 6

Student Name: Cody Jones

Student #: 5002863328

Student Email: Jonesc30@unlv.nevada.edu

Primary Github address: https://github.com/Jonesc30/Submission

Directory: Submission

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmega328PB, MPU-6050

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

#include <inttypes.h>

#include <stdlib.h>

#include <stdio.h>

#include "MPU6050_res_define.h"

#include "I2C_Master_H_file.h"

#include "USART_RS232_H_file.h"

float Acc_x,Acc_y,Acc_z,Temperature,Gyro_x,Gyro_y,Gyro_z;

void MPU6050_Init()

{

_delay_ms(150);										/* Power up time >100ms 			*/

I2C_Start_Wait(0xD0);								/* Start with device write address 	*/

I2C_Write(SMPLRT_DIV);								/* Write to sample rate register 	*/

I2C_Write(0x07);									/* 1KHz sample rate				 	*/

I2C_Stop();

I2C_Start_Wait(0xD0);

I2C_Write(PWR_MGMT_1);								/* Write to power management register 	*/

I2C_Write(0x01);									/* X axis gyroscope reference frequency */

I2C_Stop();

I2C_Start_Wait(0xD0);

I2C_Write(CONFIG);									/* Write to Configuration register */

I2C_Write(0x00);									/* Fs = 8KHz 					   */

I2C_Stop();

I2C_Start_Wait(0xD0);

I2C_Write(GYRO_CONFIG);								/* Write to Gyro configuration register */

I2C_Write(0x18);									/* Full scale range +/- 2000 degree/C 	*/

I2C_Stop();

I2C_Start_Wait(0xD0);

I2C_Write(INT_ENABLE);								/* Write to interrupt enable register */

I2C_Write(0x01);

I2C_Stop();

}

void MPU_Start_Loc()

{

I2C_Start_Wait(0xD0);								/* I2C start with device write address 			   */

I2C_Write(ACCEL_XOUT_H);							/* Write start location address from where to read */

I2C_Repeated_Start(0xD1);							/* I2C start with device read address 			   */

}

void Read_RawValue()

{

MPU_Start_Loc();									/* Read Gyro values */

Acc_x = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());

Acc_y = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());

Acc_z = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());

Gyro_x = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());

Gyro_y = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());

Gyro_z = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Nack());

I2C_Stop();

}

int main()

{

char buffer[20], float_[10];

float Xa,Ya,Za,t;

float Xg=0,Yg=0,Zg=0;

I2C_Init();

MPU6050_Init();

USART_Init(9600);

while(1)

{

Read_RawValue();

Xa = Acc_x/16384.0;								/* Divide raw value by sensitivity scale factor to get real values */

Ya = Acc_y/16384.0;

Za = Acc_z/16384.0;

Xg = Gyro_x/16.4;

Yg = Gyro_y/16.4;

Zg = Gyro_z/16.4;

dtostrf( Xa, 3, 2, float_ );					/* Takes the first parameter and formts is with size 3 precission 2 and puts it in float_ */

sprintf(buffer," Ax = %s g\t",float_);			/* This is where the value is being put as a string onto the buffer variable */

USART_SendString(buffer);						/* This is where it is being displayed on the serial terminal  */

dtostrf( Ya, 3, 2, float_ );					/* Takes the first parameter and formts is with size 3 precission 2 and puts it in float_v */

sprintf(buffer," Ay = %s g\t",float_);			/* This is where the value is being put as a string onto the buffer variable */

USART_SendString(buffer);						/* This is where it is being displayed on the serial terminal  */

dtostrf( Za, 3, 2, float_ );					/* Takes the first parameter and formts is with size 3 precission 2 and puts it in float_ */

sprintf(buffer," Az = %s g\t",float_);			/* This is where the value is being put as a string onto the buffer variable */

USART_SendString(buffer);						/* This is where it is being displayed on the serial terminal  */

dtostrf( Xg, 3, 2, float_ );					/* Takes the first parameter and formts is with size 3 precission 2 and puts it in float_ */

sprintf(buffer," Gx = %s%c/s\t",float_,0xF8);	/* This is where the value is being put as a string onto the buffer variable */

USART_SendString(buffer);						/* This is where it is being displayed on the serial terminal  */

dtostrf( Yg, 3, 2, float_ );					/* Takes the first parameter and formts is with size 3 precission 2 and puts it in float_ */

sprintf(buffer," Gy = %s%c/s\t",float_,0xF8);	/* This is where the value is being put as a string onto the buffer variable */

USART_SendString(buffer);						/* This is where it is being displayed on the serial terminal  */

dtostrf( Zg, 3, 2, float_ );					/* Takes the first parameter and formts is with size 3 precission 2 and puts it in float_ */

sprintf(buffer," Gz = %s%c/s\r\n",float_,0xF8);	/* This is where the value is being put as a string onto the buffer variable */

USART_SendString(buffer);						/* This is where it is being displayed on the serial terminal  */

_delay_ms(1000);								/* This just be a delaylay for the thingys */

}

}

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://www.youtube.com/watch?v=VzsN77IGMkg

- GITHUB LINK OF THIS DA

https://github.com/Jonesc30/Submission/tree/master/DesignAssignments

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Cody Jones