CPE301 – FALL 2019

# Midterm 1

Student Name: Cody Jones

Student #: 5002863328

Student Email: Jonesc30@unlv.nevada.edu

Primary Github address: https://github.com/Jonesc30/Submission

Directory: Submission

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmega328PB, FTDI, ESP01, LM35

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

- #define F_CPU 16000000UL

- #include <avr/io.h>

- #include <avr/interrupt.h>

- #include <stdio.h>

- #include <stdlib.h>

- #include <util/delay.h>

- #include <math.h>

- #define BAUDRATE 115200

- #define BAUD_PRESCALLER (((F_CPU / (BAUDRATE * 8UL))) - 1)

- void USART_init(void); //function to initialize USART

- unsigned char USART_receive(void); //function to receive through USART

- void USART_transmit(unsigned char data); //function to send through USART

- void USART_putstring(char* StringPtr); //function to scan through the string

- void ADC_init (void); //function to initialize ADC

- uint8_t OVF_COUNT = 0; //initialize the overflow count for interrupt

- uint8_t OVF_LIMIT = 250; //set the limit the count can reach to set 1 sec delay

- char outs[20];

- int main(void)

- {

- USART_init(); //initialize USART

- ADC_init (); //initialize ADC

- USART_putstring("AT\r\n");

- _delay_ms(500);

- USART_putstring("AT+CWMODE=3\r\n");

- _delay_ms(500);

- USART_putstring("AT+CWJAP=\"WIFI_NAME_HERE\",\"WIFI_PASSWORD_HERE\"");	// Log in WiFi

- _delay_ms(500);

- USART_putstring("AT+CIPMUX=0\r\n");

- _delay_ms(500);

- TCCR0A = 0x00; //normal operation

- TCCR0B |= (1 << CS02); //set prescalar to 256

- TCNT0 = 16; //TOP = 256-240 = 16

- TIMSK0 |= (1 << TOIE0);

- sei(); //enable interrupt

- while (1)

- { //sit here waiting for interrupt

- }

- }

- void USART_init(void)

- {

- UBRR0H = (uint8_t)(BAUD_PRESCALLER >> 8);

- UBRR0L = (uint8_t)(BAUD_PRESCALLER);

- UCSR0A = (1 << U2X0);   // double mode for more accuracy

- UCSR0B = (1 << RXEN0) | (1 << TXEN0);

- UCSR0C = (1 << UCSZ00) | (1 << UCSZ10);

- }

- unsigned char USART_receive(void)

- {

- while(!(UCSR0A & (1 << RXC0)));

- return UDR0;

- }

- void USART_transmit(unsigned char data) {			// Function to transmit ASCII value into UDR0

- while (!(UCSR0A & (1 << UDRE0)));			// Keep Checking until  UDRE0 data register 'High' to break loop

- UDR0 = data;								// Store unsigned char serial data into UDR0

- }

- void USART_putstring(char* StringPtr)

- {

- while(*StringPtr != 0x00)

- {

- USART_transmit(*StringPtr);

- StringPtr++;

- }

- }

- void ADC_init (void)

- {

- ADMUX = (0<<REFS1)|					// reference selection bits

- (1<<REFS0)|							// AVcc - external cap at AREF

- (0<<ADLAR)|							// ADC Left Adjust Result

- (1<<MUX2)|							// Analog Channel Selection Bits

- (0<<MUX1)|							// ADC4 (PC4)

- (0<<MUX0);

- ADCSRA = (1<<ADEN)|					// ADC Enable

- (0<<ADSC)|							// ADC Start Conversion

- (0<<ADATE)|							// ADC Auto Trigger Enable

- (0<<ADIF)|							// ADC Interrupt Flag

- (0<<ADIE)|							// ADC Interrupt Enable

- (1<<ADPS2)|							// ADC Prescaler Select Bits

- (0<<ADPS1)|

- (1<<ADPS0);

- }

- ISR (TIMER0_OVF_vect)

- {

- OVF_COUNT++; //increment the overflow counter

- if (OVF_COUNT == OVF_LIMIT) //check to see if the limit was reached

- {

- ADCSRA|=(1<<ADSC);	//start conversion

- while((ADCSRA&(1<<ADIF))==0);//wait for conversion to finish

- ADCSRA |= (1<<ADIF);

- int a = ADCL; //a is temperature

- a = a | (ADCH<<8);

- a = (a/1024.0) * 5000/10;

- a = (a*(2))+32; //equation to convert celsius to farenheit. use 2 instead of 9/5

- //snprintf(outs,sizeof(outs),"%3f\r\n", a);// Stores integer 'adc_temp' into the string 'outs'

- USART_putstring("AT+CIPSTART=\"TCP\",\"api.thingspeak.com\",80\r\n");	// Connect API KEy

- _delay_ms(500);

- USART_putstring("AT+CIPSEND=51\r\n");									// Send Serial Data

- _delay_ms(500);

- USART_putstring("GET /update?key=8031EGDV0KJXD8IU&field1=");// Send Value

- _delay_ms(500);

- a = a % 100;

- USART_transmit((a/10)+'0');

- a = a % 10;

- USART_transmit((a)+'0');

- _delay_ms(500);

- USART_putstring(" ");

- _delay_ms(500);

- USART_putstring("\r\n");

- USART_putstring("\r\n");

- OVF_COUNT = 0; //reset overflow counter

- }

- TCNT0 = 16; //reset TOP

- }

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://www.youtube.com/watch?v=XR4jNkcFTyQ

- GITHUB LINK OF THIS DA

https://github.com/Jonesc30/Submission/tree/master/DesignAssignments

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Cody Jones